Khajeh Nasir Toosi University of Technology

Navigation course

Homework 1

Instructor: Dr.Ali Karmozdi

Student:

Asal Sanei 40308334

Introduction

Rotation transformations exist due to the fact that no single representation is ideal for every task. Because Euler angles are easy for humans to visualize and understand—such as when we think, “turn left 30 degrees, then tilt up 15 degrees”—they work well in user interfaces, flight controls, and manual input systems. However, they are affected by gimbal lock, a phenomenon where specific orientations lead to mathematical singularities and the loss of one degree of freedom.

Quaternions elegantly address this issue using only four numbers instead of nine, making them ideal for smooth animation interpolation and spacecraft attitude control where computational efficiency and avoidance of singularities are essential.

“Leitung Cosinus” because of their mathematical simplicity for combining multiple rotations and converting vectors across coordinate frames, matrices serve as the backbone of navigation systems and sensor fusion algorithms.

Axis-angle representations offer the most geometrically intuitive description "rotate 45 degrees around this specific direction" which is useful in robotics and mechanical design, where physical rotation axes are inherent to the problem.

Due to the fact that various subsystems, software packages, and sensors commonly employ different rotation conventions in practical applications, there is a pressing requirement for conversion tools. While an aircraft's flight computer may output Euler angles for the pilot display, the attitude control system relies on quaternions to prevent gimbal lock. The star tracker sensor provides a DCM, and mission planning software specifies maneuvers using axis-angle representation. These systems cannot communicate without conversion capabilities. Moreover, some mathematical operations become significantly simpler when using certain representations: composing rotations with direction cosine matrices (DCMs) is a straightforward matrix multiplication; quaternions lend themselves well to smooth path interpolation; and Euler angles facilitate the extraction of orientation in a format that humans can easily read.

Code structure

The structure of my Matlab code is as followed:

Input

Beginning with a thorough menu-driven interface, the code directs users through four types of possible inputs: Euler angles, quaternions, DCM matrices, and axis-angle representations. For every input type, the code smartly requests all required information, including unit conversions when necessary inquiring whether angles are in degrees or radians prior to converting them to a standard internal format. When users choose Euler angles or axis-angle inputs, the code automatically converts degrees to radians to ensure that all internal calculations are performed in radians. For inputs in quaternion format, it accepts both row and column vectors, reshaping them into a uniform column format.

For DCM inputs, it builds the three-by-three matrix row by row through sequential prompts. This preprocessing ensures that regardless of how users provide their data, it enters the conversion functions in a standardized format, preventing errors and inconsistencies

Conversion functions

The core of the code consists of eight specialized conversion functions organized as sub functions at the bottom of the script. These functions form a complete conversion graph: Euler-to-DCM and Euler-to-quaternion provide direct paths from Euler angles, while DCM-to-Euler and DCM-to-quaternion extract angles or quaternions from matrices. The quaternion-to-DCM and quaternion-to-Euler functions complete the quaternion conversion paths, and axis-angle-to-DCM handles the geometric rotation representation. A critical supporting function is the skew-symmetric matrix generator, which converts a three-element vector into a special matrix form used in both quaternion-to-DCM conversion and the Rodrigues formula for axis-angle rotations.

Each conversion function implements the mathematically optimal algorithm for its specific transformation, with the DCM-to-quaternion function notably including branching logic to select the most numerically stable computation path based on the matrix trace value, preventing division by near-zero numbers that could corrupt results.

Outputs

After handling the input, the code displays a further menu inquiring users to choose their preferred output format usually providing two options fitting the type of input, like converting Euler angles to either quaternions or DCMs. After the conversion function runs, the code formats the output for presentation, with particular provisions for angle outputs that allow users to select either degree or radian representations. The display commands clearly identify what each output signifies, employing standard notation such as "Quaternion [q1; q2; q3; q4]" or "Direction Cosine Matrix (Cbn)" to prevent any misunderstanding regarding conventions.